RED HAT OPENSHIFT

Service Mesh 3.2

Ambient Mode — Deployment Guide

| Product | Red Hat OpenShift Service Mesh |
| --- | --- |
| Version | 3.2 (Sail Operator) |
| Data Plane | Istio Ambient (Sidecar-less) |
| Namespaces | istio-system · ztunnel · istio-cni · bookinfo · tracing · egress |
| Date | March 2026 |
| Author | Sunil Khobragade |
| Audience | Platform / DevOps Engineers |
| Cluster | SNO — OCP 4.20 — platform: None |

# 1. Introduction

This guide documents the deployment of Red Hat OpenShift Service Mesh 3.2 using Istio Ambient mode via the Sail Operator on a Single Node OpenShift (SNO) cluster running OCP 4.20. It covers the complete stack from initial control plane deployment through distributed tracing, Prometheus metrics collection, and service mesh observability.

## 1.1 Ambient Mode Architecture

Ambient mode eliminates sidecar injection by splitting the data plane into two layers:

- ztunnel  Zero-Trust Tunnel (ztunnel): A per-node DaemonSet deployed in the ztunnel namespace. Handles mTLS via HBONE tunneling, L4 AuthorizationPolicy enforcement, and telemetry with no changes to application pods.

- Waypoint Proxy:  An Envoy-based Gateway (istio-waypoint class) deployed per namespace for L7 policy, traffic routing, and header inspection. Required for L7 AuthorizationPolicy rules. In ambient mode, the Waypoint has istio.io/dataplane-mode=none — it is not itself enrolled in the mesh.

Figure 1 — OSSM 3.2 Ambient Mode Architecture: Control Plane, ztunnel (L4), Waypoint (L7), Application Pods, Tracing & Observability

## 1.2 Namespace Layout

| Namespace | Contents |
| --- | --- |
| istio-system | istiod control plane, ServiceMonitor for istiod |
| ztunnel | ztunnel DaemonSet, ztunnel Service, ztunnel ServiceMonitor |
| istio-cni | Istio CNI DaemonSet (node-level traffic interception) |
| bookinfo | Sample application, Waypoint, AuthorizationPolicy, HTTPRoute, PodMonitor |
| egress | Egress Waypoint, AuthorizationPolicy for external traffic control |
| tracing | TempoStack, ObjectBucketClaim, OpenTelemetryCollector, RBAC, UIPlugin |
| openshift-monitoring | cluster-monitoring-config ConfigMap (UWM enablement) |

## 1.3 Key Design Decisions

- Sail Operator: All Istio resources use apiVersion: sailoperator.io/v1. The Istio, IstioCNI, and ZTunnel CRs are managed by the Sail Operator, not istioctl install.

- Discovery Selectors: meshConfig.discoverySelectors in the Istio CR can filter istiod to only watch namespaces labeled istio-discovery: enabled. NOTE: the tracing namespace must NOT have this label — see Section 4 for details.

- Trusted ztunnel namespace: pilot.trustedZtunnelNamespace: ztunnel tells istiod which namespace hosts the ztunnel DaemonSet for secure bootstrapping.

- Single-tenant tracing: TempoStack runs in openshift tenants mode with a single dev tenant and OAuth-backed authentication. The OTEL collector routes spans to Tempo via the gateway using bearer token auth.

- Thanos/UWM for metrics: User Workload Monitoring (UWM) is already enabled. Prometheus scrapes istiod, ztunnel, and Waypoint via ServiceMonitor and PodMonitor resources.

# 2. Prerequisites

## 2.1 Operators Required

| Operator | Channel | Namespace | Purpose |
| --- | --- | --- | --- |
| Red Hat OpenShift Service Mesh (Sail) | stable | openshift-operators | Istio control plane |
| Tempo Operator | stable | openshift-operators | Distributed tracing backend |
| OpenTelemetry Operator | stable | openshift-operators | Trace collection pipeline |
| Cluster Observability Operator | stable | openshift-operators | UIPlugin for console |
| ODF / NooBaa | stable | openshift-storage | S3-compatible object storage |

oc get csv -n openshift-operators | grep -E "sail|tempo|opentelemetry|observability"

# All should show: Succeeded

## 2.2 CLI Tools

| Tool | Notes |
| --- | --- |
| oc / kubectl | OpenShift CLI for all apply/get operations |
| istioctl (v3.2) | Must match control plane version — use the version bundled with OSSM 3.2 |

# 3. Ambient Mode Deployment

Deploy components in the order below. Each step depends on the previous completing successfully.

## 3.1 Step 1 — Deploy the Istio Control Plane

File: operator/sail-operator.yaml, mesh/ambient/istio.yaml

Creates the Istio CR. Profile ambient is set, enabling ambient mode in istiod. The trustedZtunnelNamespace tells istiod which namespace to trust for ztunnel bootstrapping.

# mesh/ambient/istio.yaml

apiVersion: sailoperator.io/v1

kind: Istio

metadata:

name: default

spec:

namespace: istio-system

profile: ambient

values:

pilot:

trustedZtunnelNamespace: ztunnel

meshConfig:

enableTracing: true

extensionProviders:

- name: otel-tracing

opentelemetry:

service: otel-collector.tracing.svc.cluster.local

port: 4317

oc apply -f mesh/ambient/istio.yaml

oc get pods -n istio-system -l app=istiod

NOTE: enableTracing: true and extensionProviders must be in the Istio CR from the start. Adding them later requires a Waypoint restart to pick up the new Envoy bootstrap configuration.

## 3.2 Step 2 — Deploy Istio CNI

File: mesh/ambient/istio-cni.yaml

oc apply -f mesh/ambient/istio-cni.yaml

oc get daemonset -n istio-cni

# CNI pods must be Running on every node before deploying ztunnel

## 3.3 Step 3 — Deploy ztunnel

File: mesh/ambient/ztunnel.yaml

Creates the ZTunnel CR. The DaemonSet deploys into namespace ztunnel and handles all L4 mTLS and HBONE tunneling.

oc apply -f mesh/ambient/ztunnel.yaml

oc get pods -n ztunnel -o wide

# Every schedulable node must have a Running ztunnel pod

## 3.4 Step 4 — Enroll Namespaces

Label namespaces with istio.io/dataplane-mode=ambient to enroll them. No pod restarts required. ztunnel begins capturing traffic immediately.

oc label namespace bookinfo istio.io/dataplane-mode=ambient

# Verify

istioctl ztunnel-config workload -n bookinfo

## 3.5 Step 5 — Deploy the Waypoint

File: mesh/ambient/waypoint.yaml

Deploys a Waypoint Gateway scoped to the bookinfo namespace for L7 policy enforcement. The Waypoint has istio.io/dataplane-mode=none — it manages mesh traffic but is not itself enrolled in ztunnel.

oc apply -f mesh/ambient/waypoint.yaml

# Verify

oc get pods -n bookinfo -l gateway.istio.io/managed=istio.io-mesh-controller

oc get gateway waypoint -n bookinfo

# STATUS: Programmed = True

## 3.6 Step 6 — Apply mTLS PeerAuthentication

Files: mesh/ambient/peerauth-meshwide.yaml, mesh/ambient/peerauth-bookinfo.yaml

Two PeerAuthentication resources enforce STRICT mTLS — one globally (istio-system namespace) and one scoped to bookinfo.

oc apply -f mesh/ambient/peerauth-meshwide.yaml

oc apply -f mesh/ambient/peerauth-bookinfo.yaml

oc get peerauthentication -A

## 3.7 Step 7 — Apply AuthorizationPolicy

File: mesh/ambient/authorizationpolicy.yaml

Applies L7 AuthorizationPolicy enforced by the Waypoint. Allows only HTTP GET from specific principals to productpage.

IMPORTANT: L7 AuthorizationPolicy enforcement requires the Waypoint to be deployed and running. Without a Waypoint, L7 policies are silently not enforced.

## 3.8 Step 8 — Traffic Splitting

File: mesh/ambient/trafficroute.yaml

An HTTPRoute in the bookinfo namespace splits reviews traffic across v1/v2/v3 with configurable weights. Traffic splitting is enforced at the Waypoint (L7).

oc apply -f mesh/ambient/review-services.yaml   # reviews-v1/v2/v3 Services

oc apply -f mesh/ambient/trafficroute.yaml       # HTTPRoute with weights

# Verify

oc get httproute reviews-route -n bookinfo

# 4. Ingress Gateway

Exposes bookinfo to external traffic via Gateway API + OCP Route. An AuthorizationPolicy restricts ingress to GET-only requests.

## 4.1 Architecture

External client

-> OCP Route (reencrypt TLS termination)

-> bookinfo-gateway (Gateway API, istio gatewayClass)

-> HTTPRoute -> productpage:9080

-> AuthorizationPolicy (GET only, enforced at Waypoint)

Figure 2 — Ingress Traffic Flow: External HTTPS → OCP Route → Gateway API → ztunnel HBONE → Waypoint L7 Policy → Application Pod

## 4.2 Apply Order

oc apply -f gateways/ingress/bookinfo-namespace.yaml   # bookinfo namespace labels

oc apply -f gateways/ingress/bookinfo-gateway.yaml     # Gateway CR

oc apply -f gateways/ingress/bookinfo-httproute.yaml   # HTTPRoute

oc apply -f gateways/ingress/referencegrant.yaml       # cross-ns ReferenceGrant

oc apply -f gateways/ingress/bookinfo-route.yaml       # OCP Route

oc apply -f gateways/ingress/authpolicy.yaml           # GET-only AuthorizationPolicy

NOTE: The ReferenceGrant allows the Gateway in the ingress namespace to reference the productpage Service in the bookinfo namespace. Without it, cross-namespace routing is blocked.

# 5. Egress Gateway

Controls outbound traffic from mesh workloads to external services using a dedicated egress Waypoint and ServiceEntry. SPIFFE-based AuthorizationPolicy enforces per-workload allow/deny rules.

## 5.1 Architecture

bookinfo workload (e.g. productpage)

-> DNS capture: github.com -> synthetic 240.240.x.x mesh IP

-> ztunnel HBONE -> egress Waypoint

-> AuthorizationPolicy (SPIFFE-based ALLOW/DENY)

-> ServiceEntry -> real github.com IP

-> Internet

## 5.2 Apply Order

oc apply -f gateways/egress/egress-waypoint.yaml       # Gateway in egress namespace

oc apply -f gateways/egress/egress-serviceentry.yaml   # ServiceEntry for github.com

oc apply -f gateways/egress/egress-referencegrant.yaml # cross-ns ReferenceGrant

oc apply -f gateways/egress/egress-authpolicy.yaml     # SPIFFE-based ALLOW/DENY

NOTE: The ServiceEntry must be in istio-system (not egress namespace) for ztunnel to capture DNS and route through the egress Waypoint. A ServiceEntry in other namespaces is not picked up by ztunnel.

# 6. Distributed Tracing

Distributed tracing uses Grafana Tempo as the backend, with the OpenTelemetry Collector receiving spans from the Istio Waypoint and routing them to Tempo. Traces are exposed in the OpenShift console via a UIPlugin.

## 6.1 Architecture

bookinfo Waypoint (Envoy)

| Telemetry CR: provider=otel-tracing, sampling=100%

| OTLP gRPC -> otel-collector.tracing.svc.cluster.local:4317

v

OpenTelemetry Collector (tracing namespace)

| batch processor

| OTLP gRPC TLS + SA bearer token -> tempo-tempo-gateway:8090

v

Tempo Gateway (openshift mode, OAuth)

| SubjectAccessReview -> OCP API

| -> distributor -> ingester

v

NooBaa S3 (ODF) <- trace blocks flushed every ~5 minutes

v

OCP Console UIPlugin -> Observe -> Traces (Jaeger-compatible UI)

## 6.2 Critical Finding — tracing Namespace Must NOT Be Ambient-Enrolled

IMPORTANT: Do NOT label the tracing namespace with istio.io/dataplane-mode=ambient. Tempo uses HTTPS readiness probes (port 3101) and memberlist clustering (port 7946). ztunnel intercepts these and expects HBONE — probes fail and Tempo pods never become ready.

## 6.3 Critical Finding — No istio-discovery Label on tracing Namespace

IMPORTANT: Do NOT label the tracing namespace with istio-discovery=enabled. If this label is present, istiod registers otel-collector as a mesh service, and the bookinfo Waypoint routes to it via HBONE. Since tracing is not ambient-enrolled, the HBONE connection fails silently — no traces reach the collector.

## 6.4 Step 1 — Create Namespace and Storage

oc apply -f distributed-tracing/tracing-namespace.yaml   # NO ambient, NO istio-discovery

oc apply -f distributed-tracing/tempo/tempo-bucket.yaml  # ObjectBucketClaim

# Wait for OBC to bind

oc get obc tempo-bucket -n tracing   # PHASE: Bound

# Create tempo-s3-secret with correct key names (NooBaa uses AWS_* names,

# Tempo requires: bucket, endpoint, access_key_id, access_key_secret)

BUCKET_NAME=$(oc get cm tempo-bucket -n tracing -o jsonpath='{.data.BUCKET_NAME}')

BUCKET_HOST=$(oc get cm tempo-bucket -n tracing -o jsonpath='{.data.BUCKET_HOST}')

BUCKET_PORT=$(oc get cm tempo-bucket -n tracing -o jsonpath='{.data.BUCKET_PORT}')

ACCESS_KEY=$(oc get secret tempo-bucket -n tracing -o jsonpath='{.data.AWS_ACCESS_KEY_ID}' | base64 -d)

SECRET_KEY=$(oc get secret tempo-bucket -n tracing -o jsonpath='{.data.AWS_SECRET_ACCESS_KEY}' | base64 -d)

oc create secret generic tempo-s3-secret -n tracing \

--from-literal=bucket="${BUCKET_NAME}" \

--from-literal=endpoint="https://${BUCKET_HOST}:${BUCKET_PORT}" \

--from-literal=access_key_id="${ACCESS_KEY}" \

--from-literal=access_key_secret="${SECRET_KEY}"

## 6.5 Step 2 — Deploy TempoStack

File: distributed-tracing/tempo/tempostack.yaml

Deploys Tempo with openshift tenants mode (single dev tenant), gateway enabled, and Jaeger-compatible query frontend. The gateway provides OAuth-backed authentication via OCP.

oc apply -f distributed-tracing/tempo/tempostack.yaml

# Wait for all pods: compactor, distributor, gateway, ingester, querier, query-frontend

watch "oc get pods -n tracing"

# Verify ready

oc get tempostack tempo -n tracing   # STATUS: Ready

NOTE: TempoStack with mode: openshift requires template.gateway.enabled: true. The gateway and jaegerQuery ingress are mutually exclusive — do not set jaegerQuery.ingress when gateway is enabled. The gateway creates its own OCP Route automatically.

## 6.6 Step 3 — RBAC for OTEL Collector

File: distributed-tracing/tempo/tempo-rbac.yaml

The Tempo gateway in openshift mode uses OCP SubjectAccessReview to check if the caller's ServiceAccount has permission to write to the tenant. Without this ClusterRole the gateway returns PermissionDenied.

oc apply -f distributed-tracing/tempo/tempo-rbac.yaml

# ClusterRole: tempostack-traces-write (create on tempo.grafana.com/dev/traces)

# ClusterRole: tempostack-traces-reader (get on tempo.grafana.com/dev/traces)

# Bound to: otel-collector SA (write) + system:authenticated (read)

## 6.7 Step 4 — Deploy OTEL Collector

File: distributed-tracing/otel/collector.yaml

Receives OTLP traces from the Istio Waypoint and forwards to the Tempo gateway using bearer token authentication (pod SA token).

oc apply -f distributed-tracing/otel/collector.yaml

oc get pods -n tracing | grep otel-collector   # 1/1 Running

NOTE: The bearertokenauth extension uses the pod's Kubernetes SA token (/var/run/secrets/kubernetes.io/serviceaccount/token) as the OCP bearer token for Tempo gateway authentication.

## 6.8 Step 5 — Apply Telemetry CR

File: distributed-tracing/otel/istio-telemetry.yaml

Enables the otel-tracing extension provider for mesh-wide trace export. 100% sampling for lab environments — reduce to 1-5% in production.

oc apply -f distributed-tracing/otel/istio-telemetry.yaml

# Restart Waypoint to pick up new bootstrap config

oc rollout restart deployment/waypoint -n bookinfo

# Verify Waypoint has otel cluster

istioctl pc cluster -n bookinfo deploy/waypoint | grep otel

# otel-collector.tracing.svc.cluster.local  4317  outbound  EDS

IMPORTANT: After adding the extensionProvider to the Istio CR or applying a new Telemetry CR, the Waypoint must be restarted. The Waypoint reads tracing config at startup from the Envoy bootstrap — it does not reload dynamically.

## 6.9 Step 6 — Install UIPlugin

File: distributed-tracing/tempo/uiplugin.yaml

Installs the Cluster Observability Operator UIPlugin that embeds Tempo trace UI in OCP Console under Observe → Traces.

# First install Cluster Observability Operator

oc apply -f operator/cluster-observability-operator.yaml

# Then apply UIPlugin

oc apply -f distributed-tracing/tempo/uiplugin.yaml

# Verify

oc get uiplugin distributed-tracing   # Available = True

# OCP Console -> Observe -> Traces

# 7. Observability (Prometheus Metrics)

Metrics are scraped via ServiceMonitors and PodMonitors using OCP User Workload Monitoring (already enabled). No additional operators required. Metrics are accessible via OCP Console → Observe → Metrics.

## 7.1 Metrics Sources

| Component | Port | Monitor Type | Key Metrics |
| --- | --- | --- | --- |
| istiod | 15014 | ServiceMonitor | pilot_xds_pushes, pilot_services, pilot_proxy_convergence_time |
| ztunnel | 15020 | ServiceMonitor | workload_manager_active_proxy_count, istio_xds_message_bytes_total |
| Waypoint | 15020 + 15090 | PodMonitor | envoy_cluster_upstream_cx_active, envoy_http_downstream_rq_total |

## 7.2 Key Findings

- ztunnel has no Service: The ztunnel namespace has no Service by default. Must create a Service targeting port 15020 with label service: ztunnel for the ServiceMonitor to discover it.

- istiod Service missing metrics port: The existing istiod Service does not expose port 15014. Must create a separate Service istiod-monitor exposing port 15014 with label istio: pilot.

- Waypoint needs PodMonitor: The Waypoint Service only exposes 15008 (HBONE) and 15021 (health). Metrics ports 15020 and 15090 require a PodMonitor that selects pods directly by gateway.networking.k8s.io/gateway-name: waypoint.

- No sidecar metrics in ambient: In ambient mode, app pods have no sidecar — port 15020 does not exist on app pods. All L7 metrics come from the Waypoint, all L4 metrics from ztunnel.

- ztunnel metric names: ztunnel metrics use istio_* and workload_manager_* prefixes, not ztunnel_*. Searching for "ztunnel" in metric names returns nothing.

## 7.3 Apply Order

oc apply -f observability/monitors/istiod-monitor.yaml   # Service + ServiceMonitor

oc apply -f observability/monitors/ztunnel-monitor.yaml  # Service + ServiceMonitor

oc apply -f observability/monitors/waypoint-monitor.yaml # PodMonitor (two endpoints)

# Wait ~60s for first scrape, then verify

oc get servicemonitor istiod-monitor -n istio-system

oc get servicemonitor ztunnel-monitor -n ztunnel

oc get podmonitor waypoint-monitor -n bookinfo

## 7.4 Useful PromQL Queries

# Waypoint request rate

rate(envoy_http_downstream_rq_total{namespace="bookinfo"}[5m])

# Waypoint p99 latency

histogram_quantile(0.99, rate(envoy_http_downstream_rq_time_bucket{namespace="bookinfo"}[5m]))

# istiod xDS push rate

rate(pilot_xds_pushes[5m])

# Active workloads in ztunnel

workload_manager_active_proxy_count

# Proxy convergence p99

histogram_quantile(0.99, rate(pilot_proxy_convergence_time_bucket[5m]))

# 8. Kiali Service Mesh Observability

Kiali provides service mesh topology visualization, real-time traffic graphs, configuration validation, and trace integration. It connects to OCP User Workload Monitoring via Thanos Querier and to TempoStack for traces.

## 8.1 Key Configuration Findings

- tracing namespace label conflict: istiod uses discoverySelectors with label istio-discovery: enabled to build its service registry. If the tracing namespace has this label, HBONE routing is triggered and traces stop flowing. The tracing namespace must NOT have istio-discovery=enabled.

- Service Account Name: The operator creates kiali-service-account in istio-system — not kiali. All RBAC must reference this exact name.

- Prometheus Thanos Proxy: Use port 9091 (web) with thanos_proxy: enabled: true. Port 9092 (tenancy) returns Bad Request without namespace-scoping headers.

- Tempo Gateway URL: The correct base URL is https://tempo-tempo-gateway.tracing:8080/api/traces/v1/dev/tempo. Kiali appends /api/search, producing the working path /api/traces/v1/dev/tempo/api/search.

## 8.2 Apply Order

# Install Kiali Operator

oc apply -f operator/kiali-operator.yaml

# Apply RBAC

oc apply -f kiali/kiali-rbac.yaml

# Deploy Kiali CR

oc apply -f kiali/kiali.yaml

# Get route

oc get route kiali -n istio-system -o jsonpath='{.spec.host}'

# 9. End-to-End Validation

## 9.1 Automated Validate Script

The repository includes validate.sh which runs 60+ checks across all components.

bash validate.sh

# Expected: 60+ PASS, 0 FAIL

# WARN: otel-collector cluster on Waypoint (timing-dependent after restart)

## 9.2 Manual Checks

# Ambient mode

istioctl ztunnel-config workload -n bookinfo

oc get peerauthentication -A

# Ingress

curl -s -o /dev/null -w "%{http_code}" https://bookinfo.apps.<cluster>/productpage

# Expected: 200

# Egress

oc exec -n bookinfo deploy/productpage-v1 -- python3 -c \

"import urllib.request,ssl; r=urllib.request.urlopen(\

'https://github.com',context=ssl.create_default_context()); print(r.status)"

# Expected: 200

# Traces

for i in $(seq 1 20); do curl -s -o /dev/null https://bookinfo.apps.<cluster>/productpage; done

# OCP Console -> Observe -> Traces -> select bookinfo-gateway-istio

# Metrics

# OCP Console -> Observe -> Metrics

# Query: workload_manager_active_proxy_count

# 10. Troubleshooting

This section documents the issues encountered during deployment and their resolutions.

## 10.1 Tempo Pods Stuck at 0/1 — Readiness Probe Failures

Symptom: Tempo pods not becoming ready after enrolling tracing namespace in ambient.

Cause: Tempo uses HTTPS readiness probes on port 3101 and memberlist gossip on port 7946. ztunnel intercepts these and enforces HBONE mTLS. The Kubernetes kubelet cannot establish HBONE tunnels, so probes fail.

Fix: Remove istio.io/dataplane-mode=ambient from the tracing namespace and restart Tempo pods.

oc label namespace tracing istio.io/dataplane-mode-

oc delete pods -n tracing --all

## 10.2 No Traces Flowing — OTEL Collector Silent

Symptom: otel-collector logs completely empty. No traces in Tempo or OCP Console.

Cause: tracing namespace has istio-discovery=enabled. istiod registers otel-collector as a mesh service. Waypoint routes to it via HBONE. Since tracing is not ambient-enrolled, the receiving side drops the HBONE connection silently.

Fix: Remove istio-discovery=enabled from the tracing namespace.

oc label namespace tracing istio-discovery-

## 10.3 Tempo Gateway 504 on Query

Symptom: GET /api/traces/v1/dev/search returns 504 gateway timeout.

Cause: The Tempo gateway NetworkPolicy egress rules did not include query-frontend ports (9095, 3200) — or an incorrect TLS configuration prevented the gateway from connecting to query-frontend.

Resolution: Use the OCP Console UIPlugin (Observe → Traces) instead of the raw Tempo gateway route. The UIPlugin proxies requests correctly through the plugin pod and avoids the gateway query timeout.

## 10.4 TempoStack Secret Key Names

Symptom: TempoStack CR fails with "storage secret must contain bucket field".

Cause: NooBaa OBC creates a Secret with AWS-compatible key names (AWS_ACCESS_KEY_ID, AWS_SECRET_ACCESS_KEY). TempoStack requires different names (bucket, endpoint, access_key_id, access_key_secret).

Fix: Create a separate tempo-s3-secret with the correct key names as shown in Section 6.4.

## 10.5 Waypoint Has No otel-collector Cluster After Restart

Symptom: istioctl pc cluster -n bookinfo deploy/waypoint | grep otel returns nothing after a Waypoint restart.

Cause: The Waypoint reads tracing configuration from the Envoy bootstrap at startup. If the extensionProvider was added to the Istio CR after the Waypoint was first deployed, the Waypoint must be restarted to pick up the new bootstrap.

Fix: Restart the Waypoint and generate traffic to trigger the otel cluster to become active.

oc rollout restart deployment/waypoint -n bookinfo

# Wait for pod to restart, then generate traffic

for i in $(seq 1 10); do curl -s -o /dev/null https://bookinfo.apps.<cluster>/productpage; done

## 10.6 SNO Node Restart Recovery

Symptom: After SNO node restart, some pods have increased restart counts. Services temporarily unavailable.

Cause: On a Single Node cluster, all workloads restart simultaneously. Tempo memberlist needs time to reform, Waypoint needs to re-sync with istiod.

Recovery: Wait 2-3 minutes for all pods to stabilize. Run validate.sh to confirm all checks pass.

# Check all pods are running

oc get pods -n istio-system && oc get pods -n ztunnel

oc get pods -n bookinfo && oc get pods -n tracing

# Re-run validation

bash validate.sh

# 11. Quick Reference

## 11.1 Complete Deployment Order

| # | Command | Description |
| --- | --- | --- |
| 1 | oc apply -f operator/sail-operator.yaml | Install Sail Operator |
| 2 | oc apply -f operator/tempo-operator.yaml | Install Tempo Operator |
| 3 | oc apply -f operator/otel-operator.yaml | Install OTEL Operator |
| 4 | oc apply -f operator/cluster-observability-operator.yaml | Install COO (UIPlugin) |
| 5 | oc apply -f mesh/ambient/istio.yaml | Istio CR with tracing config |
| 6 | oc apply -f mesh/ambient/istio-cni.yaml | CNI DaemonSet |
| 7 | oc apply -f mesh/ambient/ztunnel.yaml | ztunnel DaemonSet |
| 8 | oc apply -f mesh/ambient/ (remaining) | Waypoint, PeerAuth, AuthzPolicy, HTTPRoute |
| 9 | oc apply -f gateways/ingress/ | Ingress: Gateway API + OCP Route |
| 10 | oc apply -f gateways/egress/ | Egress: Waypoint + ServiceEntry |
| 11 | Create tracing namespace (no labels) | tracing namespace |
| 12 | Create tempo-s3-secret from NooBaa OBC | S3 credentials with correct key names |
| 13 | oc apply -f distributed-tracing/tempo/ | TempoStack + RBAC + UIPlugin |
| 14 | oc apply -f distributed-tracing/otel/ | OTEL Collector + Telemetry CR |
| 15 | oc rollout restart deploy/waypoint -n bookinfo | Restart Waypoint for tracing bootstrap |
| 16 | oc apply -f observability/monitors/ | ServiceMonitor + PodMonitor |
| 17 | oc apply -f operator/kiali-operator.yaml | Kiali Operator |
| 18 | oc apply -f kiali/ | Kiali CR + RBAC |

## 11.2 Key Endpoints

| Endpoint | Purpose |
| --- | --- |
| https://bookinfo.apps.<cluster>/productpage | Bookinfo external ingress |
| https://tempo-tempo-gateway-tracing.apps.<cluster> | Tempo gateway route (use UIPlugin instead) |
| tempo-tempo-gateway.tracing.svc.cluster.local:8090 | OTEL -> Tempo (gRPC TLS + OAuth) |
| otel-collector.tracing.svc.cluster.local:4317 | Waypoint -> OTEL Collector (OTLP gRPC) |
| thanos-querier.openshift-monitoring.svc:9091 | Prometheus/Thanos for Kiali |
| https://s3.openshift-storage.svc | NooBaa S3 for Tempo storage |

## 11.3 Useful istioctl Commands

| Command | Purpose |
| --- | --- |
| istioctl version | Verify client and control plane versions match |
| istioctl proxy-status | List sync status of all proxies |
| istioctl analyze -n bookinfo | Configuration analysis |
| istioctl ztunnel-config workload -n bookinfo | Workloads captured by ztunnel |
| istioctl ztunnel-config svc --namespace ztunnel | Services with waypoint assignments |
| istioctl pc cluster -n bookinfo deploy/waypoint | Waypoint Envoy cluster config |
| istioctl pc all -n bookinfo deploy/waypoint -o json | Full Waypoint Envoy config dump |

## 11.4 Repository Structure

service-mesh-3.2/

├── operator/

│   ├── sail-operator.yaml

│   ├── tempo-operator.yaml

│   ├── otel-operator.yaml

│   └── cluster-observability-operator.yaml

├── mesh/ambient/

│   ├── istio.yaml          (includes extensionProviders + enableTracing)

│   ├── istio-cni.yaml

│   ├── ztunnel.yaml

│   ├── waypoint.yaml

│   ├── peerauth-meshwide.yaml

│   ├── peerauth-bookinfo.yaml

│   ├── authorizationpolicy.yaml

│   ├── review-services.yaml

│   └── trafficroute.yaml

├── gateways/

│   ├── ingress/

│   └── egress/

├── distributed-tracing/

│   ├── tracing-namespace.yaml   (NO ambient, NO istio-discovery)

│   ├── tempo/

│   │   ├── tempo-bucket.yaml

│   │   ├── tempostack.yaml

│   │   ├── tempo-rbac.yaml

│   │   └── uiplugin.yaml

│   └── otel/

│       ├── collector.yaml

│       └── istio-telemetry.yaml

├── observability/

│   └── monitors/

│       ├── istiod-monitor.yaml

│       ├── ztunnel-monitor.yaml

│       └── waypoint-monitor.yaml

├── kiali/

│   ├── kiali.yaml

│   └── kiali-rbac.yaml

└── validate.sh              (60+ checks)